### Models tested from HuggingFace

Functionalities:

- Analyze single image

- \\ Video Feed

- \\ Live Camera Feed

Models used:

- microsoft/Florence-2-large – 0.8B parameters

- Qwen/Qwen2.5-VL-3B-Instruct – 3 billion parameters

### Ros Bags Datasets

Functionalities:

- Extract frames from a .mcap files

- Extract and convert .mcap files into MP4

Datasets:

- https://catalog.ngc.nvidia.com/orgs/nvidia/teams/isaac/resources/r2bdataset2024?version=1

Live ROS 2 Implementation

cd ~/ros2_ws

colcon build --symlink-install

source ~/ros2_ws/install/setup.bash

source /opt/ros/humble/setup.bash

source ~/ros2_ws/install/setup.bash

ros2 launch zed_wrapper zed_camera.launch.py camera_model:=zed2i

- Running a ZED 2i camera on ROS 2 and retrieve live imagery via Python.

- Environment: Ubuntu 22.04 (Jammy) running ROS 2 Humble. (Lab Machine)

- Driver: Installed zed-ros2-wrapper and resolved build conflicts (Anaconda vs System Python) It doesn’t like running while anaconda is activated.

- Connection: Make sure it’s connected to a USB 3.0 port otherwise it will complain

- Code: Created a short Python code using cv_bridge to subscribe to a ZED topic and convert live ROS messages into OpenCV images.

GitHub Repo: https://github.com/yaman-aljnadi/Human-Computer-Interaction-Lab

Finding a way to connect this with the LLM, and talk to Cletus about it.

Creating the full workflow:

Adding the VLM into Reachy 2 and making sure that he is able to listen and speak as well as giving him some hand movements